# EPANET + WNTR Prototype Guide

## For AquaAgent — Physics-Based Water Network Simulation

Goal: Build a small but physically meaningful virtual water network that can generate pressure/flow behaviour for AquaAgent’s ML and agent layer.

## 1. What EPANET Does

EPANET is an open-source water distribution network simulator from the U.S. EPA.

It models pressurized drinking-water networks containing:

- Junctions / demand nodes

- Pipes

- Reservoirs

- Tanks

- Pumps

- Valves

It can calculate:

- Flow through pipes

- Pressure/head at nodes

- Tank levels

- Pump/valve behaviour

- Extended-period behaviour over time

- Water quality (not required for our MVP)

For AquaAgent, EPANET is the physics engine.

### Important mental model

You define the network
        ↓
EPANET solves the hydraulics
        ↓
Pressure + flow values appear
        ↓
Treat selected values as "virtual sensors"
        ↓
ML interprets those sensor signals
        ↓
AquaAgent explains + recommends action

## 2. EPANET vs WNTR

### EPANET

The actual hydraulic simulation engine / modelling environment.

Useful for:

- Building a network

- Defining pipe properties

- Running hydraulic simulations

- Viewing network behaviour

### WNTR

Water Network Tool for Resilience is a Python package from the U.S. EPA.

WNTR lets us control EPANET-style networks programmatically.

For AquaAgent, WNTR should be the main Python interface.

It can:

- Load/create EPANET networks

- Modify pipes, nodes, valves, pumps, etc.

- Change demands

- Add leaks

- Simulate failures

- Run hydraulic simulations

- Extract pressure/flow time series

- Simulate pressure-dependent demand

- Visualize results

Recommendation:

EPANET = hydraulic foundation
WNTR   = Python control layer
Python = simulation/data pipeline
ML     = anomaly interpretation
Bedrock Agent = explanation/action

# 3. What We Actually Need to Learn

You do NOT need to learn all of EPANET.

For the prototype, learn these concepts:

### Network

Understand:

Reservoir
   ↓
Junction
   ↓
Pipe
   ↓
Junction
   ↓
Demand

### Pipe

Important properties:

- Length

- Diameter

- Roughness

- Start node

- End node

### Junction

Important properties:

- Elevation

- Base demand

- Demand pattern

### Reservoir

Provides a fixed hydraulic head / water source.

### Tank

Stores water and has a changing water level.

### Valve

Controls or restricts flow.

### Pump

Adds energy/head to the network.

# 4. The Physics You Need

You don’t need to derive the entire solver.

You need to understand two ideas.

## A. Conservation of mass

At a junction:

flow in - flow out - demand = 0

Example:

10 L/s enters

3 L/s → Tap A
2 L/s → Tap B
5 L/s → downstream network

The simulator must conserve that water.

## B. Head loss

Water loses hydraulic head while flowing through pipes.

Head loss depends on things such as:

- Flow

- Pipe length

- Pipe diameter

- Pipe roughness

EPANET supports:

- Hazen-Williams

- Darcy-Weisbach

- Chezy-Manning

For the prototype, Hazen-Williams is fine initially.

Do not waste hackathon time deriving the equations yourself.

# 5. The Most Important Feature: Pressure

For AquaAgent, pressure is one of the most useful signals.

Imagine:

Normal:

Sensor A = 42 m
Sensor B = 39 m
Sensor C = 37 m

A leak occurs:

Sensor A = 40 m
Sensor B = 31 m
Sensor C = 35 m

The change is produced by the hydraulic model.

We don’t manually tell the ML model:

"there is a leak"

Instead, it receives:

timestamp
pressure_A
pressure_B
pressure_C
flow_1
flow_2
...

and has to infer the abnormal behaviour.

# 6. Modelling a Leak

A leak can be represented as an additional outlet from the network.

Conceptually:

Pipe
A ───────────────────── B
              │
              ↓
            LEAK
              ↓
           Ground

Leak discharge depends on available pressure and leak size.

Conceptually:

Leak flow ∝ leak area × √pressure

The exact formulation is handled by the simulator.

WNTR supports explicit leak modelling and pressure-dependent hydraulic simulation.

# 7. Why Pressure-Dependent Demand Matters

This is important for a realistic prototype.

In a real network, if pressure becomes too low, users may not receive the full amount of water they normally demand.

Therefore:

Pressure ↓
     ↓
Delivered demand ↓
     ↓
Network behaviour changes

This matters because a pipe leak can reduce pressure and therefore alter downstream consumption.

WNTR supports pressure-dependent demand (PDD).

For AquaAgent:

Normal pressure
     ↓
normal demand

Low pressure
     ↓
reduced delivered demand

This is more realistic than assuming every tap always consumes its full requested demand.

# 8. Extended-Period Simulation

Do NOT make the whole prototype a single static calculation.

Water demand changes with time.

Example:

08:00  morning demand ↑
10:00  demand ↓
13:00  moderate
18:00  evening demand ↑
23:00  demand ↓

EPANET can simulate the network across multiple time steps.

This gives us:

Time
 ↓
Pressure(t)
Flow(t)
Demand(t)
Tank level(t)

That is exactly what ML needs:

time-series data.

# 9. Our Prototype Network

Keep the first network tiny.

Reservoir
                    │
                  Pipe 1
                    │
                 Junction A
              ┌─────┴─────┐
              │           │
           Pipe 2      Pipe 3
              │           │
          Junction B   Junction C
           │    │        │
         Tap A Tap B    Tap C

Add 3–5 virtual sensors:

Pressure Sensor A
Pressure Sensor B
Pressure Sensor C

Flow Sensor 1
Flow Sensor 2

This is enough for the MVP.

# 10. The Interactive Demo

The frontend should control the simulation.

Possible controls:

[ Open Tap A ]
[ Close Tap A ]

[ Open Tap B ]
[ Close Tap B ]

[ Create Small Leak ]
[ Create Large Leak ]

[ Close Valve ]
[ Sensor Failure ]

The user should NOT need to understand EPANET.

They manipulate the network visually.

Behind the scenes:

Frontend
   ↓
Python API
   ↓
WNTR
   ↓
Hydraulic simulation
   ↓
Virtual sensor data
   ↓
ML model

# 11. The Key Hackathon Experiment

Do not simply demonstrate:

Click leak → AI says leak.

Instead:

### Experiment

- Start normal network.

- Open/close several taps.

- Change demand.

- Secretly introduce a leak.

- Generate sensor observations.

- Feed only observations to ML.

- Ask ML to identify the anomaly.

- Estimate the probable network zone.

- Send the incident to AquaAgent.

- Agent explains the event and recommends an action.

The judge should feel like they are trying to fool the system.

# 12. Scenarios to Generate

Generate labelled simulation data for:

| Scenario | Expected behaviour |
| --- | --- |
| Normal | Baseline |
| High demand | Flow ↑, pressure may ↓ |
| Small leak | Local pressure/flow disturbance |
| Large leak | Stronger disturbance |
| Pipe burst | Extreme disturbance |
| Valve closure | Topology/pressure changes |
| Sensor failure | Bad sensor signal |
| Low reservoir level | System-wide pressure changes |

This prevents the model from learning the useless rule:

pressure ↓ = leak

Instead, it learns to distinguish multiple causes of abnormal behaviour.

# 13. Generating Training Data

Randomise the simulation.

For every run, vary:

Demand
Leak location
Leak size
Valve state
Reservoir level
Pipe properties
Sensor noise
Time of day

Example:

Run 001
Leak: none
Demand: normal

Run 002
Leak: Pipe 4
Size: small
Demand: normal

Run 003
Leak: Pipe 2
Size: large
Demand: high

Run 004
Valve: closed
Leak: none
Demand: normal

Save the resulting sensor time series.

Example dataset:

timestamp
pressure_1
pressure_2
pressure_3
flow_1
flow_2
demand_1
demand_2
scenario
fault_location
fault_severity

# 14. ML MVP

Do NOT start with a giant neural network.

First establish a baseline.

Possible models:

### Option 1 — Random Forest

Use for:

Normal
High demand
Leak
Burst
Valve closure
Sensor fault

### Option 2 — Isolation Forest

Use when you mainly want:

Normal vs abnormal

### Option 3 — Autoencoder

Use later for:

learn normal hydraulic behaviour
        ↓
large reconstruction error
        ↓
anomaly

Start simple.

The novelty should come from the whole system, not from unnecessarily complicated ML.

# 15. Leak Localisation

Detection:

Something abnormal is happening.

Localisation:

The abnormality is probably around Branch 3.

This is much more valuable.

For the MVP, use:

Sensor pattern
      ↓
Compare against simulated fault signatures
      ↓
Rank probable locations

Example:

1. Branch 3 — 81%
2. Branch 4 — 13%
3. Branch 2 — 6%

Call this:

probable leakage zone

not:

exact pipe location

unless you actually validate exact localisation.

# 16. Virtual Sensors

You do not need to physically install sensors.

EPANET/WNTR generates the physical state.

We select certain nodes/links as if sensors were installed there.

Example:

Reservoir
                 │
              [Flow S1]
                 │
            Junction A
          [Pressure S1]
            /        \
        Pipe 2      Pipe 3
          │            │
   [Pressure S2]  [Pressure S3]

The ML model only receives these selected measurements.

This creates the sparse sensing problem.

# 17. Why Sparse Sensors Matter

Don’t give ML every node.

That makes the problem trivial.

Instead:

50 network nodes
       ↓
5 sensors
       ↓
infer what is happening

That creates the real challenge:

Can a small number of measurements reveal where the hydraulic abnormality is occurring?

That is a much better research story.

# 18. Sensor Noise

Perfect simulation is dangerous.

Real sensors aren’t perfect.

Add controlled noise:

true pressure = 40.0 m

sensor reading:
39.7
40.2
39.8
40.4

Also simulate:

- missing readings

- temporary spikes

- sensor drift

- stuck values

Then test whether the model remains useful.

# 19. Calibration — Later, Not MVP

A real digital twin would ideally be calibrated against field measurements.

Conceptually:

Real network
     ↓
Real sensor data
     ↓
Compare
     ↓
Simulation
     ↓
Adjust model parameters

For the hackathon, you probably won’t have enough real municipal data.

Therefore:

Be transparent that the prototype uses a simulated network and synthetic sensor data.

Do not pretend the Pune dashboard represents live PMC telemetry.

# 20. Python Setup

Recommended environment:

python -m venv .venv

Activate it, then:

pip install wntr pandas numpy matplotlib scikit-learn

WNTR can run EPANET simulations through its EpanetSimulator.

Basic structure:

import wntr

wn = wntr.network.WaterNetworkModel("network.inp")

sim = wntr.sim.EpanetSimulator(wn)

results = sim.run_sim()

pressure = results.node["pressure"]
flow = results.link["flowrate"]

The exact network file comes later.

# 21. First Coding Milestone

Before building React, AWS or ML:

### Goal

Get this working:

Python
  ↓
WNTR
  ↓
Load network
  ↓
Run simulation
  ↓
Print pressure
  ↓
Print flow

Then:

Add leak
  ↓
Run simulation
  ↓
Observe changed pressure/flow

If this works, the foundation is solid.

# 22. Second Milestone

Generate many simulations.

WNTR
 ↓
1000 scenarios
 ↓
sensor readings
 ↓
CSV

Then inspect:

pressure vs time
flow vs time

before touching ML.

# 23. Third Milestone

Train:

sensor data
     ↓
ML classifier
     ↓
scenario

Then test on simulations the model has never seen.

# 24. Fourth Milestone

Add localisation:

sensor pattern
     ↓
probable fault zone

# 25. Fifth Milestone

Connect:

ML output
     ↓
Bedrock
     ↓
AquaAgent

Example:

Incident detected: abnormal pressure behaviour
Likely cause: underground leak
Probable zone: Branch 3
Estimated severity: Medium
Recommended action: inspect downstream valve chamber

# 26. Final Architecture

┌──────────────────┐
                  │  Interactive UI  │
                  └────────┬─────────┘
                           │
                           ▼
                    Simulation API
                           │
                           ▼
                 ┌──────────────────┐
                 │    WNTR/EPANET   │
                 │ Hydraulic Twin   │
                 └────────┬─────────┘
                          │
                   Virtual sensors
                          │
                          ▼
                 ┌──────────────────┐
                 │   ML Detection   │
                 └────────┬─────────┘
                          │
                    anomaly + class
                          │
                          ▼
                 ┌──────────────────┐
                 │  Localisation    │
                 └────────┬─────────┘
                          │
                    probable zone
                          │
                          ▼
                 ┌──────────────────┐
                 │  AquaAgent /     │
                 │  Bedrock Agent   │
                 └────────┬─────────┘
                          │
                          ▼
                  Explanation + Action

# 27. What NOT to Build

For the first prototype, skip:

- Water-quality modelling

- Chemical reactions

- Full Pune network reconstruction

- 3D pipe rendering

- Exact real-world pipe coordinates

- Huge deep-learning models

- Real sensor hardware everywhere

- RL unless the response-optimisation problem is actually ready

### Build this first:

Small network → realistic hydraulics → hidden fault → sparse virtual sensors → ML → localisation → agent response.

# 28. Official References

EPANET — U.S. EPA

EPANET 2.2 User Manual

EPANET 2.2 GitHub

WNTR Documentation

WNTR GitHub

WNTR Getting Started

WNTR Hydraulic Simulation

EPANET 2.2 User Manual — EPA Text Version

# 29. The Learning Order

Don’t try to learn everything at once.

DAY 1
EPANET basics
↓
nodes / pipes / reservoir / demand

DAY 2
Hydraulics
↓
pressure / flow / head loss

DAY 3
WNTR
↓
load + modify + simulate network

DAY 4
Leaks
↓
create controlled leak scenarios

DAY 5
Time series
↓
generate sensor data

DAY 6
ML
↓
normal vs anomaly

DAY 7+
Localisation + AWS + AquaAgent

## The single most important principle

EPANET/WNTR should generate the physics. Your ML should never invent the physics.

That separation is what makes AquaAgent technically credible.