# AquaAgent — Physics-Based Simulation Specification v1

## Purpose

Build Phase 1 only of AquaAgent: a physics-based digital twin of a small pressurized water-distribution network.

The simulator will later become the ground-truth generator for:

Physics simulation
      ↓
Graph ML prediction
      ↓
Anomaly detection
      ↓
Optional RL decision layer
      ↓
Explainable AI / AquaAgent

Do not implement ML, RL, AWS, Bedrock, or the final UI in Phase 1.

## 1. Recommended Starting Network

Use the official EPANET 2.2 tutorial example network as the baseline.

The EPA example has:

- 1 reservoir

- 6 junctions

- 1 tank

- 8 pipes

- 1 pump

- a two-loop distribution network

- a tank connected to the network

That gives 8 total nodes, which is almost exactly the desired 5–10-node scale. The EPA documentation provides the node/pipe properties, pump design point, pressure-driven example, and extended-period example. citeturn3search1turn2view0

### Baseline nodes

| Node | Type | Elevation | Base demand |
| --- | --- | --- | --- |
| 1 | Reservoir | 700 ft | 0 gpm |
| 2 | Junction | 700 ft | 0 gpm |
| 3 | Junction | 710 ft | 150 gpm |
| 4 | Junction | 700 ft | 150 gpm |
| 5 | Junction | 650 ft | 200 gpm |
| 6 | Junction | 700 ft | 150 gpm |
| 7 | Junction | 700 ft | 0 gpm |
| 8 | Tank | 830 ft | 0 gpm |

### Baseline pipes

| Pipe | Length | Diameter | C-factor |
| --- | --- | --- | --- |
| 1 | 3000 ft | 14 in | 100 |
| 2 | 5000 ft | 12 in | 100 |
| 3 | 5000 ft | 8 in | 100 |
| 4 | 5000 ft | 8 in | 100 |
| 5 | 5000 ft | 8 in | 100 |
| 6 | 7000 ft | 10 in | 100 |
| 7 | 5000 ft | 6 in | 100 |
| 8 | 7000 ft | 6 in | 100 |

The official tutorial uses Hazen-Williams head loss and these values. citeturn3search1

### Pump

Use the documented pump design point:

- 600 gpm

- 150 ft head

### Tank

Use the documented tank:

- elevation ≈ 830 ft

- initial depth ≈ 3.5–4 ft

- maximum depth = 20 ft

- diameter = 60 ft

The exact initial-depth value should follow the chosen EPANET 2.2 reference version. citeturn3search1

### Official references

EPANET 2.2 Quick Start — Example Network

EPANET Official Tutorial

EPANET 2.2 User Manual

WNTR Documentation

WNTR GitHub

# 2. Why This Network

Do not start with a completely artificial pipe layout.

The documented example is small, looped and contains branching, different elevations, multiple demands, a pump and a tank.

That gives enough hydraulic complexity for the experiment while remaining completely inspectable.

WNTR’s official examples also span from simple 9-node networks to much larger systems. citeturn0search1

The official EPANET tutorial explicitly uses this network for:

- snapshot hydraulic analysis

- pressure-driven analysis

- extended-period simulation

- time-varying demand

so it is a particularly good baseline for our first prototype. citeturn1view0

# 3. Core Research Idea

The simulator knows the entire network.

The future ML system sees only a sparse subset.

FULL PHYSICS WORLD
                 EPANET / WNTR
                       │
          ┌────────────┴────────────┐
          │                         │
          ▼                         ▼
     Ground truth             Sparse observations
     ALL nodes/pipes           3 main sensors
          │                         │
          │                         ▼
          │                    Future ML
          │                         │
          └──────────────► Evaluation

For example:

Full network:

N1 ─ N2 ─ N3 ─ N4
      │    │
      N5 ─ N6
           │
           N7
           │
           N8

The simulator knows every state.

The ML model may only receive:

Sensor N2
Sensor N4
Sensor N6

The goal is eventually to infer unobserved hydraulic states.

# 4. Important Refinement of the ML Idea

Do not build a future model around fixed node IDs.

Avoid:

N2 + N4 + N6 → N7

Instead represent the network as a graph.

### Node features

elevation
demand
pressure
head
node type
observed/unobserved mask

### Edge features

length
diameter
roughness
status
flow

Then a future graph/message-passing model can learn:

observed nodes
+
topology
+
physical parameters
        ↓
unobserved-node prediction

This makes the idea compatible with different child configurations.

# 5. Child → Grandchild Prediction

The eventual research experiment can be:

Observed parent nodes
        ↓
predict child
        ↓
predicted child becomes context
        ↓
predict grandchild
        ↓
continue

Example:

Sensor
  ↓
Child 1
  ↓
Child 2
  ↓
Child 3

But do not assume accuracy remains constant.

Measure error by graph distance:

1-hop error
2-hop error
3-hop error
4-hop error

This creates a useful research question:

How far can hydraulic state be propagated from sparse observations before prediction error becomes unacceptable?

The simulator must preserve graph distance so this can be evaluated later.

# 6. Full Physics State

For every timestep, retain as much of the following as possible.

### Node

timestamp
node_id
node_type
elevation
pressure
head
base_demand
delivered_demand

### Pipe

timestamp
pipe_id
start_node
end_node
flow
status
length
diameter
roughness

### Tank

timestamp
tank_level
tank_head

### Pump

timestamp
pump_flow
pump_head
pump_status

### Hidden scenario state

simulation_id
scenario_type
fault_type
fault_location
fault_start
fault_end
leak_area
leak_flow
severity
random_seed
network_configuration_id

# 7. Hydraulic Modes

Support both:

### Demand-driven analysis

wn.options.hydraulic.demand_model = "DD"

Useful for normal operating conditions.

### Pressure-dependent demand

wn.options.hydraulic.demand_model = "PDD"

Important for low-pressure and leak scenarios.

WNTR documents both modes and recommends pressure-dependent demand when low pressure means consumers may not receive their requested demand. citeturn1search1

# 8. Leak Simulation

Never simulate a leak by manually doing:

pressure -= 10

That destroys the physics.

Use WNTR’s leak model.

WNTR models leak discharge as pressure-dependent; its documented form includes:

Q = Cd × A × √(2gh)

where leak flow depends on leak area and hydraulic pressure/head. WNTR can also insert a leak along a pipe by splitting the pipe and creating a leak node. citeturn1search1turn1search2

Generate:

- small leak

- medium leak

- large leak

- burst

- different locations

- different start times

- different durations

# 9. Non-Leak Scenarios

This is mandatory.

Otherwise the future ML model can learn:

pressure ↓ = leak

Generate:

- Normal operation

- High demand

- Low demand

- Demand redistribution

- Valve closure

- Pump variation

- Low reservoir head

- Tank-level variation

- Sensor failure

- Small leak

- Medium leak

- Large leak

- Burst

This allows the future anomaly detector to distinguish different causes of abnormal hydraulic behaviour.

# 10. Extended-Period Simulation

Use time-varying demand.

The official EPANET tutorial demonstrates a 24-hour diurnal demand pattern over a 72-hour extended-period simulation. It notes that real deployments should use site-specific demand data. citeturn2view0

For synthetic training:

night       → low
morning     → rising
day         → moderate
evening     → high
night       → falling

Randomise:

- peak magnitude

- peak time

- minimum demand

- node-specific demand multiplier

- weekday/weekend-like profiles

Do not use exactly one demand curve for all training samples.

# 11. Sensor Layer

The physics engine exposes all nodes.

Create a separate sensor layer.

Default experiment:

3 pressure sensors
+
1–2 flow sensors

Example:

sensor_nodes = ["2", "4", "6"]
sensor_links = ["3", "6"]

Sensor placement must be configurable.

Generate multiple placement experiments.

Example:

[2,4,6]
[3,5,7]
[2,5,8]

Later compare prediction quality against sensor placement.

# 12. Sensor Noise

Generate both:

### True value

pressure_true
flow_true

### Observed value

pressure_measured
flow_measured

Optional corruption:

- Gaussian noise

- missing readings

- spikes

- bias

- slow drift

- stuck sensor

Always preserve the clean value.

# 13. Small-Network Generalisation

The 8-node EPA network is the baseline, not the only network.

After the baseline works, generate a family of 5–10-node networks.

Vary:

number of junctions
branch depth
loop presence
pipe lengths
diameters
roughness
elevations
demands
reservoir head
tank level
sensor locations

Keep the networks hydraulically valid.

The future model should learn physical/topological relationships rather than memorising one graph.

# 14. Dataset Generation

Each simulation is one complete sample/episode.

Do NOT randomly split individual timesteps from the same simulation between train and test.

Instead:

70% simulations → train
15% simulations → validation
15% simulations → test

Prefer an even harder test:

Training:
many configurations

Validation:
new parameter combinations

Test:
unseen network configurations
+
unseen fault locations

This tests actual generalisation.

# 15. Dataset Tables

### node_states

simulation_id
timestamp
node_id
elevation
pressure
head
base_demand
delivered_demand

### pipe_states

simulation_id
timestamp
pipe_id
start_node
end_node
flow
status
length
diameter
roughness

### sensors

simulation_id
timestamp
sensor_id
source_node_or_link
measurement_type
true_value
measured_value

### scenarios

simulation_id
scenario_type
fault_type
fault_location
fault_size
fault_start
fault_end
random_seed
network_configuration_id

### graph

network_configuration_id
node_id
node_features
edge_id
start_node
end_node
edge_features

# 16. Validation

Every simulation must check:

- hydraulic convergence

- network connectivity

- mass balance

- valid tank level

- valid pressure/head

- valid pipe status

- valid demand

- no accidental missing components

Failed simulations should be logged rather than silently included.

# 17. Future ML Target

The simulator should make this future problem possible:

INPUT

Sparse observed node states
+
network topology
+
node physical features
+
edge physical features
+
time/context

              ↓

      Graph-based model

              ↓

OUTPUT

Pressure of unobserved nodes
Head of unobserved nodes
Demand of unobserved nodes
Flow of relevant edges

Candidate architectures later:

- Message Passing Neural Network

- GNN

- Graph Attention Network

- Graph Transformer

- Physics-informed GNN

Do not choose one until the physics dataset exists.

# 18. Future Anomaly Detection

Later:

Sparse observations
       ↓
ML predicted state
       ↓
Observed vs predicted
       ↓
Residual
       ↓
Anomaly score

Example:

Predicted pressure = 38.5 m
Observed pressure  = 31.0 m

Residual = 7.5 m

This residual can become one signal for the anomaly detector.

# 19. Future RL Layer

RL should not be used just because the project needs RL.

The natural place for RL is after anomaly detection.

### State

network condition
suspected incidents
pressure
flow
tank level
service status

### Actions

inspect zone
isolate branch
close valve
reduce pressure
dispatch crew
repair leak

### Reward

water saved
+ service maintained
- repair cost
- pressure violations
- customer impact

The future RL problem becomes:

Which intervention should the municipality perform first under limited resources?

WNTR also provides documented capabilities for disruptive incidents and response/repair strategies, supporting this later direction. citeturn0search17

# 20. Future Explainable AI

The final AquaAgent should explain:

WHAT happened?
WHY is it suspicious?
WHERE is it likely happening?
WHAT evidence supports this?
WHAT action is recommended?

The LLM/agent must consume structured model outputs.

It must never invent hydraulic measurements.

# 21. Phase 1 Deliverables

Phase 1 is complete when the system can:

- ☐ Build/load the 8-node EPA baseline network

- ☐ Run hydraulic simulation

- ☐ Run extended-period simulation

- ☐ Run pressure-dependent demand

- ☐ Create physics-based leaks

- ☐ Create non-leak disturbance scenarios

- ☐ Generate full-state data

- ☐ Generate sparse sensor data

- ☐ Add configurable sensor noise

- ☐ Generate hundreds/thousands of simulations

- ☐ Preserve ground truth

- ☐ Reproduce simulations from seeds

- ☐ Visualise pressure/flow/tank behaviour

- ☐ Export ML-ready datasets

# 22. What NOT to Build Yet

Do not build:

- AWS

- Bedrock

- RL

- React dashboard

- Pune map

- city-scale network

- physical sensors

- complicated deep-learning models

- water-quality modelling

First prove:

The hydraulic world behaves correctly.

# 23. Suggested Project Structure

aquaagent/
│
├── config/
│   ├── network.yaml
│   └── simulation.yaml
│
├── network/
│   ├── build.py
│   └── topology.py
│
├── scenarios/
│   ├── demand.py
│   ├── leaks.py
│   ├── valves.py
│   └── generator.py
│
├── simulation/
│   ├── hydraulic.py
│   └── runner.py
│
├── sensors/
│   └── virtual.py
│
├── dataset/
│   └── export.py
│
├── analysis/
│   └── plots.py
│
├── data/
│
├── notebooks/
│
├── requirements.txt
└── README.md

# 24. MASTER PROMPT FOR A CODING AGENT

Copy everything below into a coding model/agent:

## Prompt

You are a scientific Python engineer specialising in hydraulic water-distribution simulation.

Build Phase 1 of AquaAgent, a physics-based digital twin and synthetic-data generator for a small pressurised water-distribution network.

### Absolute priority

Create a correct physics-based simulation first.

Do NOT implement:

- machine learning

- reinforcement learning

- AWS

- Bedrock

- LLMs

- frontend/dashboard

Those are later phases.

### 1. Baseline network

Use the official EPANET 2.2 tutorial example network as the starting configuration.

It contains:

- reservoir Node 1

- junction Nodes 2–7

- tank Node 8

- Pipes 1–8

- Pump Link 9

- Hazen-Williams head-loss formulation

Use the official documented:

- node elevations

- base demands

- pipe lengths

- pipe diameters

- roughness/C-factor

- pump design point

- tank properties

Do not invent replacement values unless required for compatibility.

Source:

https://usepa.github.io/EPANET2.2/2_quickstart.html

### 2. Technology

Use:

Python
WNTR
EPANET 2.2-compatible hydraulics
NumPy
Pandas
Matplotlib

Install WNTR with:

pip install wntr

Use WNTR’s WaterNetworkModel and hydraulic simulators.

Prefer EpanetSimulator for EPANET-compatible hydraulic results.

Use WNTRSimulator where its leak/PDD functionality is needed.

The implementation must clearly document which simulator is used for each experiment.

### 3. Network abstraction

Create a reusable graph representation.

Each node must expose:

id
type
elevation
base demand
coordinates

Each pipe must expose:

id
start node
end node
length
diameter
roughness
status

Do not hard-code the future ML architecture around node IDs.

### 4. Hydraulic simulation

Support:

snapshot simulation
extended-period simulation
demand-driven analysis
pressure-dependent demand analysis

The simulator must output:

node pressure
node head
node demand
pipe flow
tank level
pump state

### 5. Time-varying demand

Implement configurable 24-hour demand patterns.

Randomise:

peak magnitude
peak timing
node multiplier
minimum demand

Support simulations of:

24 hours
48 hours
72 hours

### 6. Physics-based leak generation

Implement real hydraulic leaks.

NEVER do:

pressure -= arbitrary_value

Use WNTR’s leak model.

Support:

leak location
leak area
leak start time
leak end time
discharge coefficient

Support leaks at junctions and at arbitrary positions along pipes where WNTR allows this.

Store exact leak ground truth.

### 7. Fault/scenario generator

Generate:

NORMAL
HIGH_DEMAND
LOW_DEMAND
DEMAND_SHIFT
VALVE_CLOSURE
PUMP_VARIATION
LOW_RESERVOIR_HEAD
TANK_VARIATION
SMALL_LEAK
MEDIUM_LEAK
LARGE_LEAK
PIPE_BURST
SENSOR_FAULT

Each scenario gets:

simulation_id
random_seed
scenario_type
fault_type
fault_location
fault_size
fault_start
fault_end
network_configuration_id

### 8. Sparse sensors

The simulator has complete access to every node.

Create a configurable sparse observation layer.

Default:

3 pressure sensors
1–2 flow sensors

Example:

sensor_nodes = ["2", "4", "6"]

The model-facing dataset must NOT contain the hidden state of unobserved nodes.

However, the ground-truth dataset must retain everything for evaluation.

Create:

full_state
sparse_observation

as separate outputs.

### 9. Sensor noise

Implement optional:

Gaussian noise
missing measurements
spikes
bias
drift
stuck sensor

Store:

true_value
measured_value

Do not overwrite true physics.

### 10. Network variation

After validating the baseline network, create optional generators for related small networks with:

5–10 nodes

Vary:

branching
loop presence
branch depth
pipe length
pipe diameter
roughness
elevation
demand
reservoir head
tank level

All generated networks must pass hydraulic validation.

### 11. Future ML compatibility

Do not implement ML.

But prepare the data for a future graph model.

Create graph tensors/records containing:

node features
edge features
connectivity
observed-node mask

The future model should eventually learn:

sparse observed nodes
+
network topology
+
physical parameters
        ↓
unobserved-node hydraulic state

The future model should not depend on fixed node IDs.

### 12. Recursive prediction experiment support

Store graph distance between observed and unobserved nodes.

This will later allow:

1-hop prediction
2-hop prediction
3-hop prediction

to be evaluated separately.

Do not assume recursive predictions remain accurate.

### 13. Dataset schema

Create:

node_states

simulation_id
timestamp
node_id
elevation
pressure
head
base_demand
delivered_demand

pipe_states

simulation_id
timestamp
pipe_id
start_node
end_node
flow
status
length
diameter
roughness

sensors

simulation_id
timestamp
sensor_id
source
measurement_type
true_value
measured_value

scenarios

simulation_id
scenario_type
fault_type
fault_location
fault_size
fault_start
fault_end
random_seed
network_configuration_id

### 14. Dataset splitting

Never split individual timesteps randomly across train/test.

Split by complete simulations.

Use:

70% train
15% validation
15% test

Also provide an optional hard-generalisation split where test simulations contain:

unseen network configurations
unseen fault locations
unseen parameter combinations

### 15. Validation

Every simulation must verify:

hydraulic convergence
network connectivity
mass balance
valid tank level
valid pressure/head
valid demand

Failed runs must be logged.

Never silently save failed simulations as training data.

### 16. Demonstration

Create one deterministic demo:

- Run the network normally.

- Introduce a hidden leak.

- Continue simulation.

- Show full physics behaviour.

- Show pressure/flow changes.

- Hide the leak label.

- Show only the sparse sensor measurements.

- Save the exact hidden leak metadata separately.

The future ML model must receive observations, not the leak label.

### 17. Visualisations

Generate:

network topology
pressure over time
flow over time
tank level over time
demand over time
sensor readings
leak event timeline

Also create a plot comparing:

true pressure
vs
measured pressure

### 18. Reproducibility

Every run must be reproducible from:

network configuration
simulation configuration
random seed

Save the configuration used for each dataset.

### 19. Documentation

Create a README explaining:

- EPANET

- WNTR

- network topology

- hydraulic assumptions

- demand model

- leak model

- sensor model

- dataset schema

- how to run one simulation

- how to generate 100 simulations

- how to reproduce a simulation

- limitations

Use official EPA/WNTR documentation as references.

### 20. Final acceptance test

The implementation is accepted only if it can:

Load/build the EPA 8-node network
        ↓
Run hydraulic simulation
        ↓
Generate time-series states
        ↓
Introduce a physics-based leak
        ↓
Observe resulting pressure/flow changes
        ↓
Generate sparse sensor observations
        ↓
Export full ground truth + sparse observations
        ↓
Repeat this for many randomized scenarios

Do not proceed to ML until this pipeline is reliable.

# 25. Research Direction After Phase 1

Once the simulator works, the development order should be:

PHASE 1
Physics simulation
        ↓
PHASE 2
Graph-based hydraulic state prediction
        ↓
PHASE 3
Prediction-residual anomaly detection
        ↓
PHASE 4
Leak localisation
        ↓
PHASE 5
RL-based intervention prioritisation
        ↓
PHASE 6
Explainable AquaAgent
        ↓
PHASE 7
AWS deployment + polished hackathon product

The key scientific experiment is not merely:

“Can ML detect a leak?”

It should eventually become:

“How much hydraulic state can be reconstructed from sparse observations, how does prediction degrade with graph distance, and can the resulting residuals identify abnormal network behaviour?”

That gives the project a much stronger technical backbone than simply training a leak classifier.